Answer the following in the space provided.

/1

/3

/3

/1

/1

/1

/1

/1

/1

/4

/1

/1

/3

A material that allows electrons to move freely through it is a _______________________________.

- What three factors are required to create an electrical circuit?

- Name the unit used to measure each of the following:

Current
Voltage
Resistance

- What type of fire extinguisher should be used to fight an electrical fire?

- What type of meter can be used to measure voltage, resistance and current?

- What is done to the handles of pliers to help prevent electrical shock?

- What are the best type of plier for bending loops in the end of wires?

- A material that allows some electron movement is a ______________________________.

- If a device has no tubes, but has various other electrical components in it, it is often referred to as being ____________________________________.

- Power(P) is measured in Watts (w). The formula for power is Power=Voltage x Current (P=VI) solve the following:
a) V=6v, I=24A, P=__________________		b)V=115v, I=15A, P=____________________

c) V=_____________, I=0.5A, P=600w		d) V=1.5v, I=______________, P=24w

- In an electrical circuit, what is the purpose of having a protection device?

- What is the load in the circuit in a hand held flashlight?

- Ohm’s law says that Current equals Voltage divided by Resistance. Use this formula to solve the following:

a) Current = 15 amps, Resistance = 1200 ohms, and Voltage = ___________________________

b) Current = _____________________, Resistance = 4000 ohms, and Voltage = 12 volts

c) Current = 5mA, Resistance = __________________________, Voltage = 1.5 volts

/2

/1

/2

/1

/5

/6

Name two mechanical connectors.

- How are most household wires connected?

- What is the purpose of flux and where is it generally found in electronic solder?

- ________________________ is the process of heating a soldering iron, applying solder and wiping it smooth.

- Draw a circuit which has two bulbs and a switch wired in parallel.

/39

Draw a circuit with three bulbs wired in series with a switch.